ARM Instruction set quick ref.: www.arm.com, http://simplemachines.it/doc/QRC0001H_rvct_v2.1_arm.pdf

http://infocenter.arm.com/help/topic/com.arm.doc.qrc0001l/QRC0001_UAL.pdf

| Arithmetic | ADD{condition}{S} Rd, Rn, <Operand> |
| --- | --- |
|  | ADC{condition}{S} Rd, Rn, <Operand> |
|  | SUB{condition}{S} Rd, Rn, <Operand> |
|  | SBC{condition}{S} Rd, Rn, <Operand> |
|  | RSB{condition}{S} Rd, Rn, <Operand> |
|  | RSC{condition}{S} Rd, Rn, <Operand> |
|  | MUL{condition}{S} Rd, Rm, Rs |
|  | MLA{condition}{S} Rd, Rm, Rs, Rn |
|  | UMULL{condition}{S} RdLo, RdHi, Rm, Rs |
|  | UMLAL{condition}{S} RdLo, RdHi, Rm, Rs |
|  | SMULL{condition}{S} RdLo, RdHi, Rm, Rs |
|  | SMLAL{condition}{S} RdLo, RdHi, Rm, Rs |
| Move | MOV{condition}{S} Rd, <Operand> |
|  | MVN{condition}{S} Rd, <Operand> |
| Logical | TST{condition} Rn, <Operand> |
|  | TEQ{condition} Rn, <Operand> |
|  | AND{condition}{S} Rd, Rn, <Operand> |
|  | EOR{condition}{S} Rd, Rn, <Operand> |
|  | ORR{condition}{S} Rd, Rn, <Operand> |
|  | BIC{condition}{S} Rd, Rn, <Operand> |
| Compare | CMP{condition} Rn, <Operand> |
|  | CMN{condition} Rn, <Operand> |
| Branch | B{condition} label |
|  | BL{condition} label |
| Load / Store | LDR{condition} Rd, <Addressing_mode1> |
|  | STR{condition} Rd, <Addressing_mode1> |
| Stack | LDM{condition}<Addressing_mode2> Rn{!}, <register_list> |
|  | STM{condition}<Addressing_mode2> Rn{!}, <register_list> |
| Software Interrupt | SWI{condition} #<immediate_value> |

{S} means : Updates condition flags if S present

Required fields

| <Operand> |  |  |
| --- | --- | --- |
| Immediate value | #<immediate_value> |  |
| Register | Rm |  |
| Logical shift left | Rm, LSL #<shift> |  |
| Logical shift right | Rm, LSR #<shift> |  |
| Arithmetic shift right | Rm, ASR #<shift> |  |
| Rotate right | Rm, ROR #<shift> |  |
| <Addressing_mode1> |  |  |
| Zero offset |  | [Rn] |
| Immediate offset |  | [Rn, #+/-< immediate_value>] |
| Register offset |  | [Rn, +/-Rm] |
| Post-indexed Immediate offset |  | [Rn], #+/-< immediate_value> |
| Post-indexed Register offset |  | [Rn], +/-Rm |
| Pre-indexed Immediate offset |  | [Rn, #+/-< immediate_value>]! |
| Pre-indexed Register offset |  | [Rn, +/-Rm]! |
| <Addressing_mode2> |  |  |
| Full Descending |  | FD |
| Empty Descending |  | ED |
| Full Ascending |  | FA |
| Empty Ascending |  | EA |

Optional fields {condition}

| EQ | Equal to zero (Z=1) |  | HI | Unsigned higher |
| --- | --- | --- | --- | --- |
| NE | Not equal to zero (Z=0) |  | LS | Unsigned lower or same |
| MI | Negative |  | GE | Signed greater than or equal |
| PL | Positive or zero |  | LT | Signed less than |
| VS | Overflow |  | GT | Signed greater than |
| VC | No overflow |  | LE | Signed less than or equal |